## Generating Electricity

Conduct the experiment by following the steps listed below.  Record your data on the table provided for you.  Then, graph your data in the box labeled “Figure 1.”  Lastly, answer the questions about your experiment.

Procedure

- Wind the wire around the tube about 20 times

- Your teacher will show you how to attach both ends of the wire to the voltmeter.

- Move the magnet near the coil, but not through it.  Observe the voltage produced on the voltmeter.

- Move the magnet in various directions around the coil and observe the voltage produced.

- Place the magnet inside the coil and move it back and forth.

- Next, move the coil while holding the magnet stationary.

- Now get out your stopwatch and begin watching the seconds.  Move the magnet back and forth 1 time per second and record the voltage produced.

- Repeat the above step at 2, 3 and 4 times per second and record the voltage produced.

- Move the magnet as fast as you can and record the maximum voltage produced.

| Speed of Magnet | 1x per sec. | 2x per sec. | 3x per sec. | 4x per sec. | Max Speed |
| --- | --- | --- | --- | --- | --- |
| Voltage Produced |  |  |  |  |  |

Voltage (V)

1x

2x

3x

4x

Max

Figure 1. The amount of electricity produced as the speed of the magnet moving inside the coil increased.

Questions

- To make electricity, a ________________ and a ________________ must move back and forth past each other.

- When these two parts are put together, the resulting machine is called a ____________________________.

- When the meter made the greatest movement, in what direction were you moving the magnet?

- Was there a difference between moving the magnet through the coil and moving the coil over the magnet?

- As you moved the magnet faster, what happened to the voltage produced? Why?

- How is a turbine used at power plants to make electricity?

- Name all of the types of energy that turn a turbine to make electricity.  Which of these sources of energy are renewable?